CocoaGuard

An IoT‑Based Intrusion Detection System for Cocoa Farm Protection; Simulation Study Using PIR Sensors, GSM Alerts, and Local Deterrence

1. Problem Statement

Cocoa farmers in Uganda face significant financial losses due to theft of mature cocoa pods. Thieves often operate at night or when the farmer is away. Traditional security measures like fencing or watchmen are either too expensive or ineffective.

CocoaGuard is an affordable, real‑time intrusion detection system that:

- Detects human presence using a PIR motion sensor

- Scares off intruders immediately with a loud buzzer and flashing LED

- Sends an SMS alert to the farmer’s mobile phone

- Operates autonomously on solar power without mains electricity

- Reduces false alarms (e.g., from animals or wind) using smart filtering

2. Target Users

Primary User: Smallholder cocoa farmers who want to protect their harvest without high costs or constant physical presence.

Secondary Users: Farm workers, neighbors, or community watch groups who can respond to alerts. The system can also notify a family member.

3. Sensors and Devices Involved

The proposed system uses low‑cost, readily available components, all of which can be simulated during the ideation phase:

| Component | Purpose | Simulation Equivalent |
| --- | --- | --- |
| PIR Motion Sensor | Detects human movement by sensing infrared radiation | Virtual PIR sensor in Tinkercad with a clickable button to simulate motion. |
| GSM Module (SIM800L) | Sends SMS alerts via the mobile network to the farmer’s phone | Virtual SIM800L in Tinkercad that outputs AT commands to the Serial Monitor. |
| Microcontroller (Arduino Uno) | Reads sensor input, processes data, and controls the GSM module | Virtual SIM800L in Tinkercad that outputs AT commands to the Serial Monitor |
| Solar Panel + Battery | Provides autonomous power for continuous operation in the field (no mains electricity). | Potentiometer simulates battery voltage |
| Buzzer | Loud sound to scare thief | Virtual buzzer (or just digital pin toggling) |
| High-brightness LED | Flashing light to deter | Virtual LED |
| Potentiometer | Simulates varying battery voltage | Tinkercad analog potentiometer |

4. Data to be Collected

During simulation, the system will generate and “collect” the following data, which can be recorded from the Serial Monitor:

Motion Events: Each time the virtual PIR sensor is triggered, a timestamp (simulated using millis() or manually noted) and a message “Motion detected!” are printed

SMS Alert Log: The AT commands sent to the GSM module and the responses are printed, showing that an SMS would have been sent (e.g., AT+CMGS="+256..." and the message content).

System Status: Startup messages, sensor state changes (motion ended), and any error messages.

Data format (CSV logged from Serial Monitor):

| Field | Example |
| --- | --- |
| timestamp_ms | 1250 |
| event_type | MOTION_START, SMS_SENT, LOW_BATTERY |
| sensor_id | PIR1, GSM, BATTERY |
| value | 1 (motion), 5.2 (volts) |
| alert_sent | TRUE / FALSE |
| sms_status | SUCCESS, FAIL, COOLDOWN |
| battery_voltage | 4.8 |

This dataset will be used to create visualisations and a Streamlit dashboard.

5. Proposed Project Architecture

The architecture is divided into two phases: simulation (current ideation stage) and real deployment (future implementation)

5.1 Simulation Architecture (Tinkercad)

- User inputs:

  - Click a button to simulate PIR motion.

  - Turn a potentiometer to change battery voltage.

- Logic steps :

  - False-alarm filter: Two button presses within 5 seconds are required to confirm an intrusion.

  - Cooldown: After one SMS, no further SMS for 10 seconds.

  - Local deterrence: Upon confirmed intrusion, buzzer sounds and LED flashes for 10 seconds.

  - Battery check: Every 60 seconds, read the potentiometer (simulated battery voltage). If below 4.5V, send a “Low battery” SMS.

- Output: CSV lines printed to Serial Monitor. Copy text to file.

5.2 Real Deployment Architecture (Future)

- Power: 10W solar panel + 12V rechargeable battery + charge controller.

- Detection: One or two PIR sensors. The two-detection rule is implemented in software.

- Deterrence: 100dB+ piezo buzzer and a high-flux LED mounted on a pole.

- Alert: SIM800L sends SMS to farmer and optionally to a neighbour.

- Data logging (optional): SD card module stores events for later analysis.

Data Flow (for Simulation)

Start

│

▼

Read PIR (button)

│

▼

First detection? ──No──> Wait & loop

│Yes

▼

Start 5-sec timer

│

▼

Second detection within 5 sec? ──No──> Log FALSE_ALARM, reset

│Yes

▼

CONFIRMED INTRUSION

│

├─► Turn on buzzer & flashing LED for 10 sec

├─► Check cooldown (last SMS >10 sec ago?)

│     │Yes ──► Send SMS via GSM, log SMS_SENT

│     └─No──► Log ALERT_BLOCKED (cooldown)

│

▼

Continue reading PIR; every 60 seconds read battery

│

▼

If battery < threshold → Send LOW_BATTERY SMS

This architecture is simple, scalable (multiple sensors can be added), and fully testable in simulation before any hardware purchase.

## 6. Bill of Materials (for Real Deployment, estimated costs)

| Component | Approx. Cost (USD) |
| --- | --- |
| Arduino Uno (or Nano) | 5-10 |
| PIR sensor (HC-SR501) | 2–3 |
| SIM800L GSM module | 10–15 |
| Active buzzer (100dB) | 1–2 |
| High-brightness LED + resistor | 1 |
| 12V solar panel (10W) | 15–20 |
| 12V rechargeable battery (7Ah) | 15–20 |
| Solar charge controller | 5–8 |
| Wires, enclosure, SIM card | 5–10 |
| Total estimated | 60–90 |

## 7. Simulation Implementation Steps

- Create a new Tinkercad project with Arduino Uno, push button, buzzer, LED, virtual SIM800L, and a potentiometer.

- Write the Arduino code implementing:

  - Two-detection false-alarm filter.

  - 10-second cooldown.

  - Buzzer/LED control.

  - Battery voltage reading every 60 seconds.

  - CSV logging.

- Run simulation scenarios:

  - Valid intrusion (two presses within 5 seconds).

  - False alarm (single press).

  - Repeated presses to test cooldown.

  - Low battery (turn potentiometer down).

- Copy Serial Monitor output to run1.txt, run2.txt, etc.

- Run a Python parser to convert logs to intrusion_events.csv.

- Analyse the data using pandas/matplotlib (average intrusion duration, false alarm reduction, cooldown effectiveness).

- Build a Streamlit dashboard showing timeline, alert success rate, and battery status.

## 8. Expected Outcomes

- A fully tested simulation of CocoaGuard – a practical farm security system.

- A structured dataset that proves the system’s logic works.

- Evidence that the two-detection filter reduces false alarms and the cooldown prevents SMS flooding.

- An interactive dashboard to visualise system performance.

- A complete design ready for real-world construction (BOM, wiring diagram, and code).

9. Conclusion

CocoaGuard transforms a simple intrusion detection idea into a robust, affordable, and testable solution for cocoa farmers. By adding active deterrence, false-alarm filtering, and battery monitoring, the system becomes much more practical for field deployment.

The simulation-first approach using Tinkercad allows complete testing and data collection before any hardware purchase – saving time and money. The dataset and dashboard provide a professional proof of concept that can be presented to potential users, funders, or academic supervisors.

## CocoaGuard Project Folder Structure

CocoaGuard/

│

├── 01_simulation/

│   ├── Tinkercad_code/

│   │   ├── CocoaGuard.ino          # Main Arduino code (with all logic)

│   │   └── diagram.json            # Tinkercad wiring (components & connections)

│   │

│   ├── data_logs/

│   │   └── raw/                    # Store raw Serial Monitor outputs here

│   │       ├── run1_short.txt

│   │       ├── run2_false_alarm.txt

│   │       ├── run3_repeated.txt

│   │       └── run4_low_battery.txt

│   │

│   └── scripts/

│       └── parse_logs.py           # Python script to convert raw logs to CSV

│

├── 02_dataset/

│   ├── intrusion_events.csv        # Final cleaned dataset (all runs combined)

│   ├── battery_log.csv             # Optional: extracted battery readings

│   └── metadata.txt                # Simulation parameters (cooldown, threshold, etc.)

│

├── 03_analysis/

│   ├── exploration.ipynb           # Jupyter notebook for data analysis

│   ├── plots/                      # Saved graphs (PNG, PDF)

│   │   ├── intrusion_timeline.png

│   │   ├── false_alarm_reduction.png

│   │   └── sms_success_rate.png

│   └── analysis_report.pdf         # Exported summary (optional)

│

├── 04_dashboard/

│   ├── app.py                      # Streamlit dashboard code

│   ├── requirements.txt            # Python packages needed (streamlit, pandas, etc.)

│   └── assets/

│       └── style.css               # Optional custom styling

│

├── 05_real_hardware/

│   ├── bom.xlsx                    # Bill of materials with prices & links

│   ├── wiring_diagram.png          # Fritzing or hand-drawn wiring

│   ├── real_code.ino               # Modified code for real hardware (low power, etc.)

│   └── calibration_notes.txt       # PIR sensitivity, GSM antenna placement

│

├── README.md                       # Project overview for someone new

└── LICENSE                         # (Optional) e.g., MIT or GPL

## What Each Folder Contains (Simple Explanation)

| Folder | Purpose |
| --- | --- |
| 01_simulation | All files for the Tinkercad simulation: Arduino code, wiring diagram, raw logs from Serial Monitor, and a Python script to clean those logs. |
| 02_dataset | The final, clean CSV files ready for analysis. Also a metadata file that describes how you ran the simulation (cooldown value, false-alarm window, etc.). |
| 03_analysis | Jupyter notebook where you explore the data, create plots, and answer your research questions. |
| 04_dashboard | The Streamlit web app that loads the CSV and shows interactive charts. |
| 05_real_hardware | When you decide to build the real system, this folder keeps the parts list, wiring diagram, and adapted code (e.g., with sleep modes to save battery). |
| README.md | A single page explaining the project, how to run the simulation, and how to reproduce the analysis. |